ECPS204 Project2-Multi-threading canny edge

CHENG-CHIH, LEE, 29329351

JONATHAN KIM, 42411903

Project Description (2 pts)

- The purpose of this project is to implement multithreading on the canny application. OpenMP and pthread are explored and implemented.

- The improvement of performance is expected to be measured and discussed.

- Experimental Setup (4 pts)

For this project, we set up a Git repository to exchange files between the RPi and laptop. Following the instructions, we get each step done and recorded.

The local canny file is used to study multithreading method. And the developed canny.cpp in proj1 is also modified to implement multithreading.

- Results (6 pts)

Step 2 – Understanding the modifications on canny_local.c with OpenMP

| canny_local_omp.c | canny_local_.c |
| --- | --- |
| In Blur_x, omp is implemented to create 4 threads. omp_set_dynamic(0) configured omp to not changing thread numbers dynamically unless limited by hardware resource. | None |
| Parallel variable copies are created for each thread. Otherwise, they will be shared. | None |
| #pragma omp for… dertermins the for loop to be run separately in each thread. “r” is copied here, | None |

Step 3 – Compiling and running the modified code

#canny_local_omp

real	0m0.782s

user	0m1.006s

sys	0m0.032s

#canny_local

real	0m0.992s

user	0m0.902s

sys	0m0.020s

From the results, we can see with multithreading implementation, the real run time is shortened. However, the user and system time increased. The real time is the wall time; the user time is the total cpu time and system time is the kernel time. With multithreading, the wall time is improved and cpu/system time increased due to multithread/core execution.

Step 4 – Use OpenMP to parallelize Blur in the y direction

OpenMP is implemented for Blur Y shown as above. In BlurY multithreading implementation, n_thread isn’t declared again since it was done before BlurX. The variables to be copied are (r, rr, dot, sum).

tempim and kernel are freed after all the threads are done execution.

From this snapshot, we can see there are also 4 threads created for Blur Y.

And from the time result below, we can tell the real time is improved by approximately 33% compared to the original application without multithreading and 15 % compared to the application which only implemented multithreading in BlurX.

real	0m0.662s

user	0m0.913s

sys	0m0.044s

Step 6 – Understanding the modifications on canny_local.c with pthread

| canny_local_pthread.c | canny_local_.c |
| --- | --- |
| Structure is created for passing arguments for multithreading | None |
| blur_x is modified to take argument and have local variables declared with init value from the argument, | None |
| Number of thread is defined as 4 | None |
| In gaussian smooth, threads are created for BlurX and executed | None |
| Each thread is waited to join before process moves on. | None |

Step 7 – Compiling and running the modified code with pthreads

#canny_pthread

real	0m0.852s

user	0m0.954s

sys	0m0.032s

#canny_local_omp

real	0m0.782s

user	0m1.006s

sys	0m0.032s

#canny_local

real	0m0.992s

user	0m0.902s

sys	0m0.020s

From the results, we can tell the implementation of pthread did improve the wall time, but not as much as OpenMP.

Step 8 – Modifying the Blur in the y - direction with pthreads

Arguments added for Blur Y use.

Blur Y seperated for multithreading. Iteration on rows, so we modified the for loops as cols in the inner loop.

Threads created for Blur Y.

Now from the snapshot, we can see there are 4 threads created for Blur Y.

Time results are below:

#canny_pthread X+Y

real	0m0.530s

user	0m0.779s

sys	0m0.032s

#canny_local_omp

real	0m0.662s

user	0m0.913s

sys	0m0.044s

#canny_local

real	0m0.992s

user	0m0.902s

sys	0m0.020s

We can see the wall time is improved and having better performance than omp.

Step 9 – Comparing OpenMP and pthreads

|  | Original | OMP | OMP to Original | pThread | pThread to Original |
| --- | --- | --- | --- | --- | --- |
| real | 0m0.992s | 0m0.662s | Improve 33.3% | 0m0.530s | Improve 46.6% |
| user | 0m0.902s | 0m0.913s | Deteriorate 1.2% | 0m0.779s | Improve 13.6% |
| sys | 0m0.020s | 0m0.044s | Deteriorate 120% | 0m0.032s | Deteriorate 60% |

From the comparison, we can see that OpenMP is bringing large overhead to the kernel loading. And pthread is also resulting in better WALL time improvement.

Step 10 - Applying multi-threading (in OpenMP and pthread individually) to the camera-capturing-and-edge-detection system (in Proj #1-b)

- Problems and Discussion (6 pts)

First, we were a little stuck by the camera library but that was an easy fix by switching to libcamera.

Setting up the repository took us a little bit of time since we would need the passkey to operate git access on the RPi.

Other parts are fine and clear.

- Conclusion (2 pts)

After setting up the environment and RPi, the camera functioned as expected.

After testing canny with different parameters, we came up with the conclusion that the <sigma> defines the magnitude of smoothing; <tlow> defines the lower threshold to remove an edge; <thigh> defines the higher threshold to keep an edge.